Bluetooth communication protocol

Applicable instrument: HW8076

Table of contents

Instruction set

1. Bluetooth service UUID

2.Device Information服务

3. Transparent service

The app reads the scooter operation data script 0xA0

The app reads the scooter BMS data script 0xA1

The mobile phone sends the command code to read the skateboard information 0xB0

The APP issues the control data script 0xC0

Controller MCU upgrade

Request an upgrade script 0xD0

擦除MCU flash指令码 0xD1

The upgrade is completed 0xD3 the script

4.AT Instruction Service

Bluetooth Broadcast Name Query and Settings

Set/query encryption mode

Set/look up passwords

5. VCU upgrade service

Read VCU firmware version information

Send the upgrade package information

Send the upgrade file

CRC

Data encryption

Encryption

Encrypted byte description

Modify the record

| Protocol version | Date | Modify the content |
| --- | --- | --- |
| V1.0 | 2022-01-17 | First release |
| V1.1 | 2022-02-25 | Added VCU upgrade, added AT commands related to changing passwords, and changed the SN code acquisition method |
| V1.2 | 2022-09-13 | Modified cruise control speed limits |
| V1.3 | 2022-10-13 | Modified the software version number on page 11 and the software version number on page 21 BYTE4 and BYTE5 |
| V1.4 | 2023-01-30 | Modified the MCU upgrade protocol |
| V1.5 | 2023-03-04 | Added a MAC address acquisition interface |

Glossary

| Acronym | Definition |
| --- | --- |
| BMS | Battery Management System |
| MCU | Motor Controller Uint |
| RPM | Revolution Per Minute |
| SN | Serial Number |
| VCU | Vehicle Control Unit |
| CMD | Command |

# Instruction set

| Script | Description | Note: |
| --- | --- | --- |
| 0xA0 | Read the scooter operation data instructions | The app does not read the meter will also be automatically uploaded |
| 0xA1 | Read battery data instructions | The app does not read the meter will also be automatically uploaded |

| Script | Description | Note: |
| --- | --- | --- |
| 0xB0 | Read version information and model instructions | Meters are not automatically uploaded |

| Script | Description | Note: |
| --- | --- | --- |
| 0xC0 | APP writes instructions | Set function switch signals, speed limit values, etc |

| Script | Description | Note: |
| --- | --- | --- |
| 0xD0 | Query if the controller allows upgrades |  |
| 0xD1 | Erase the controller flash command |  |
| 0xD2 | Upgrade data |  |
| 0xD3 | The upgrade is complete |  |

# 1. Bluetooth service UUID

The above picture shows all the service channels of our Bluetooth chip, in this project we need to use 3 of them in addition to Device Information, namely:

UUID：0000fff0-0000-1000-8000-00805f9b34fb

UUID：0000f2f0-0000-1000-8000-00805f9b34fb

UUID：f000ffc0-0451-4000-b000-000000000000

The communication framework diagram is as follows:

# 2.Device Information服务

SN码通过Device Information的Serial Number String获取

SN code is in ascii code format, 13 characters

MAC通过Device Information的System ID获取

MAC is in ascii code format, 12 characters

# 3. Transparent service

透传服务UUID：0000fff0-0000-1000-8000-00805f9b34fb
	透传接收UUID：0000fff1-0000-1000-8000-00805f9b34fb（write）
	透传发送UUID:0000fff2-0000-1000-8000-00805f9b34fb(notify)

The app can obtain scooter operation data, BMS data, control scooter status and upgrade controllers through this service.

The frame diagram is as follows:

## The app reads the scooter operation data script 0xA0

The app does not need to send this command to read the data, because the meter will actively upload this frame data at regular intervals.

Example:

Write:	F0 A0 05 C9 F0

Notify:	F0 A0 19 00 00 00 00 19 10 19 0A 00 00 10 00 C8 19 19 00 C8 00 00 00 03 4F

The parse of each byte looks like this:

app sends:

| Serial number | Byte definition |  | Example |
| --- | --- | --- | --- |
| 0 | Baotou |  | 0XF0 |
| 1 | Script |  | 0xA0 |
| 2 | Total number of bytes in the frame |  | 0x05 |
| 3 | CRC16 low bytes |  | 0xC9 |
| 4 | CRC16 High Bytes |  | 0xF0 |

Meter reply: (This package meter will be actively uploaded to the app at a scheduled time of 200ms).

| Serial number | Byte definition | Example | Content definition | Note: |
| --- | --- | --- | --- | --- |
| 0 | Baotou | 0XF0 |  |  |
| 1 | Script | 0xA0 |  |  |
| 2 | Total number of bytes in the frame | 0x19 |  | 0x19=25 |
| 3 | Fault code high bytes | 0x00 |  | See definitions below |
| 4 | Fault code low byte | 0x00 |  | See definitions below |
| 5 | Control function high bytes | 0x00 |  | See definitions below |
| 6 | Control function low byte | 0x00 |  | See definitions below |
| 7 | Cruise control speed setting | 0x19 | 5-25km/h can enter the fixed speed cycle | 0x19=25km/h |
| 8 | Current speed | 0x10 |  | 0x10=16km/h |
| 9 | Maximum speed limit | 0x19 | Maximum speed 25km/h | 0x19=25km/h |
| 10 | Subtotal mileage | 0x0A | Mileage of this ride | 0x0A=10km |
| 11 | Total mileage is high bytes | 0x00 |  | Km |
| 12 | Total mileage is low bytes | 0x00 |  | Km |
| 13 | Remaining range | 0x10 | Based on the power estimate | 0x10=16km |
| 14 | The current limit value is high bytes | 0x00 | Unit 0.1A | 0x00C8=200 represents 20A |
| 15 | The current limit value is low bytes | 0xC8 |  |  |
| 16 | Current motor temperature | 0x19 | Range 0-255°C | 25℃ |
| 17 | Current controller temperature | 0x19 | Range 0-255°C | 25℃ |
| 18 | The current motor speed is high in bytes | 0x00 | Units RPM | 200RPM |
| 19 | The current motor speed is low bytes | 0xC8 |  |  |
| 20 | Retained | 0x00 |  |  |
| 21 | Retained | 0x00 |  |  |
| 22 | Retained | 0x00 |  |  |
| 23 | CRC16 low bytes | CRCL |  | MODBUS——CRC16 |
| 24 | CRC16 High Bytes | CRCH |  |  |

Control function byte definition:

| Bit7-Bit6 | Bit5 | Bit4 | Bit3-Bit2 | Bit1-Bit0 |
| --- | --- | --- | --- | --- |
| Turn signals: 0x0: Off 0x1: Turn right 0x2: Turn left | Cruise control switch: 0: Off 1: On | Headlights: 0: Off 1: On | Retained | Gear: 0x0: 1 gear 0x1: 2 gears 0x2: Gear 3 |

| Bit15-Bit11 | Bit10 | Bit9 | Bit8 |
| --- | --- | --- | --- |
| Retained | Boot mode: 0: Glide start 1: Zero start | Unit: 0:Km 1:Mile | Locked status: 0: Unlocked 1: Locked |

Trouble code byte definition:

| Fault code | Description |
| --- | --- |
| Bit0 |  |
| Bit1 | 0: Normal 1: Brake finger derailment is faulty |
| Bit2 | 0: Normal 1: Throttle finger flip fault |
| Bit3 | 0: Normal 1: Instrument and controller communication failure |
| Bit4 | 0: Normal 1: Controller overcurrent |
| Bit5 | Retained |
| Bit6 | Retained |
| Bit7 | 0: Normal 1: Hall fault |
| Bit8 | Retained |
| Bit9 | 0: Normal 1: Op amp bias fault |
| Bit10-bit15 | To be defined |

E0: Motor Temp Out of RangeE1: Brake FaultE2: Throttle FaultE3: Communication FaultE4 Overcurrent FaultE5: Undervoltage FaultE6: Overvoltage Fault

E7: Motor Hall FaultE8: Motor Phase Fault
E9: MCU Over TemperatureE10: MCU Temp Out of RangeE11: Motor Over TemperatureE12: Bluetooth FaultE13: Battery Fault

## The app reads the scooter BMS data script 0xA1

The app does not need to send this command to read the data, because the meter will actively upload this frame data at regular intervals.

Example:

Write:	F0 A1 05 C8 60

Notify:	F0 A1 19 00 00 00 0A 01 A4 64 00 10 0F A0 0F A0 19 00 00 00 00 00 00 A2 C0

The parse of each byte looks like this:

app sends:

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xA1 |
| 2 | Total number of bytes in the frame | 0x05 |
| 3 | CRC16 low bytes | 0xC8 |
| 4 | CRC16 High Bytes | 0x60 |

Meter reply: (This package meter will be actively uploaded to the app at a scheduled time of 200ms).

| Serial number | Byte definition | Example | Content definition | Note: |
| --- | --- | --- | --- | --- |
| 0 | Baotou | 0XF0 |  |  |
| 1 | Script | 0xA1 |  |  |
| 2 | Total number of bytes in the frame | 0x19 |  |  |
| 3 | The battery status is high bytes | 0x00 |  | See definitions below |
| 4 | Battery status low bytes | 0x00 |  |  |
| 5 | The battery current is high in bytes | 0x00 | Symbolized unit 0.1A | 0x000A = 10 1.0A |
| 6 | Low battery current | 0x0A |  |  |
| 7 | The current voltage of the battery is high bytes | 0x01 | Unit 0.1V | 0x01A4 = 420 42.0V |
| 8 | The current voltage of the battery is low bytes | 0xA4 |  |  |
| 9 | Current battery level | 0x64 | Unit 1% | 100% |
| 10 | The number of charging cycles is high | 0x00 |  | 0x0010 = 16 times |
| 11 | The number of charging cycles is low bytes | 0x10 |  |  |
| 12 | The battery capacity is high bytes | 0x0F | Unit mAh |  |
| 13 | Battery capacity is low bytes | 0xA0 |  |  |
| 14 | The remaining capacity is high bytes | 0x0F | Unit mAh |  |
| 15 | Low bytes of remaining capacity | 0xA0 |  |  |
| 16 | Battery temperature | 0x19 | There is a sign unit of °C | 0x19 = 25 25℃ |
| 17 | Retained | 0x00 |  |  |
| 18 | Retained | 0x00 |  |  |
| 19 | Retained | 0x00 |  |  |
| 20 | Retained | 0x00 |  |  |
| 21 | Retained | 0x00 |  |  |
| 22 | Retained | 0x00 |  |  |
| 23 | CRC16 low bytes | CRCL |  | MODBUS——CRC16 |
| 24 | CRC16 High Bytes | CRCH |  |  |

Note: Instruction 0xA#packet app can read without issuing commands The meter will automatically poll and upload these data

Battery status byte definition:

| bit15-bit1 | bit0 |
| --- | --- |
| Retained | Charging flag: 1 - Charged 0 - Not charged |
| Bit32-bit16 |  |
| Retained |  |

## The mobile phone sends the command code to read the skateboard information 0xB0

Example:

Write:	F0 B0 05 C4 30

Notify:	F0 B0 20 00 01 30 30 30 30 30 30 30 30 30 30 30 30 30 00 00 00 10 10 20 20 30 30 00 		00 00 F6 9E

The parse of each byte looks like this:

app sends:

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xB0 |
| 2 | Total number of bytes in the frame | 0x05 |
| 3 | CRC16 low bytes | 0xC4 |
| 4 | CRC16 High Bytes | 0x30 |

Meter reply:

| Serial number | Byte definition | Example | Content definition | Note: |
| --- | --- | --- | --- | --- |
| 0 | Baotou | 0XF0 |  |  |
| 1 | Script | 0xB0 |  |  |
| 2 | Total number of bytes in the frame | 0x19 |  | 0x19=25 |
| 3 | The model is high bytes | 0x00 | Model 0x0001=S008 Base |  |
| 4 | Model low byte | 0x01 |  |  |
| 5 | Instrument hardware version | 0x10 | Indicates version 1.0 | V1.0 |
| 6 | Instrument software version | 0x22 | Indicates version 2.2 | V2.2 |
| 7 | Controller hardware version | 0x20 | Indicates version 2.0 | V2.0 |
| 8 | Controller software version | 0x20 | Indicates version 2.0 | V2.0 |
| 9 | BMS hardware version | 0x30 | Indicates version 3.0 | V3.0 |
| 10 | BMS software version | 0x30 | Indicates version 3.0 | V3.0 |
| 11 | Retained | 0x00 |  |  |
| 12 | Retained | 0x00 |  |  |
| 13 | Retained | 0x00 |  |  |
| 14 | Retained | 0x00 |  |  |
| 15 | Retained | 0x00 |  |  |
| 16 | Retained | 0x00 |  |  |
| 17 | Retained | 0x00 |  |  |
| 18 | Retained | 0x00 |  |  |
| 19 | Retained | 0x00 |  |  |
| 20 | Retained | 0x00 |  |  |
| 21 | Retained | 0x00 |  |  |
| 22 | Retained | 0x00 |  |  |
| 23 | CRC16 low bytes | CRCL |  |  |
| 24 | CRC16 High Bytes | CRCH |  |  |

## The APP issues the control data script 0xC0

The app can be used to control the scooter gear, headlight switch, cruise control switch, lock status, unit, start mode, speed limit value, etc.

Example:

Write:	F0 C0 0F 00 00 19 19 00 00 00 00 00 00 39 E8

Notify:	F0 C0 06 00 30 B8

The parse of each byte looks like this:

app sends:

| Serial number | Byte definition | Example | Content definition | Note: |
| --- | --- | --- | --- | --- |
| 0 | Baotou | 0XF0 |  |  |
| 1 | Script | 0xC0 |  |  |
| 2 | Total number of bytes in the frame | 0x0F |  | 0x0A=10 |
| 3 | Control function high bytes | 0x00 |  |  |
| 4 | Control function low byte | 0x00 |  |  |
| 5 | Cruise control speed setting | 0x05 | Speed>=5km/h can enter the fixed speed cycle | 0x05=5km/h |
| 6 | Maximum speed limit | 0x19 | Maximum speed 25km/h | 0x19=25km/h |
| 7 | Retained | 0x00 |  |  |
| 8 | Retained | 0x00 |  |  |
| 9 | Retained | 0x00 |  |  |
| 10 | Retained | 0x00 |  |  |
| 11 | Retained | 0x00 |  |  |
| 12 | Retained | 0x00 |  |  |
| 13 | CRC16 low bytes | CRCL |  | MODBUS——CRC16 |
| 14 | CRC16 High Bytes | CRCH |  |  |

Data Reception Success Meter Response:

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xC0 |
| 2 | Total number of bytes in the frame | 0x06 |
| 3 | State bytes | 0x00 |
| 4 | CRC16 low bytes | CRCL |
| 5 | CRC16 High Bytes | CRCH |

Status byte: 0x00 --data received successfully 0x01--CRC check-error

Control function byte definition:

| Bit7-Bit6 | Bit5 | Bit4 | Bit3-Bit2 | Bit1-Bit0 |
| --- | --- | --- | --- | --- |
|  | Cruise control switch: 0: Off 1: On | Headlights: 0: Off 1: On | Retained | Gear: 0x0: 1 gear 0x1: 2 gears 0x2: Gear 3 |

| Bit15-Bit11 | Bit10 | Bit9 | Bit8 |
| --- | --- | --- | --- |
| Retained | Boot mode: 0: Glide start 1: Zero start | Unit: 0:Km 1:Mile | Locked status: 0: Unlocked 1: Locked |

## Controller MCU upgrade

During the upgrade process, if the instruction does not reply in 1s, resend it, and the upgrade is considered to have failed if the number of sends exceeds 5 times

MCU upgrade flow chart:

N

And

N

And

N

N

And									 And

N

And									   N

Y                                 N

And

And

Before upgrading, you need to compare the hardware version of the MCU with the hardware version of the upgrade file, and the hardware version can only be upgraded if the hardware version is the same.

The maximum number of reoccurrences was 5 times, with a time interval of 1s.

## Request an upgrade script 0xD0

App sending:

| Serial number | Byte definition |  |
| --- | --- | --- |
| 0 | Baotou | 0xF0 |
| 1 | Script | 0xD0 |
| 2 | CRC16 low bytes | 0x44 |
| 3 | CRC16 High Bytes | 0x2C |

Meter reply:

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xD0 |
| 2 | State bytes | 0x00 |
| 3 | CRC16 low bytes | CRCL |
| 4 | CRC16 High Bytes | CRCH |

Status bytes: 0x00 - Upgrade allowed 0x01 - Upgrade not allowed

## 擦除MCU flash指令码 0xD1

App sending:

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xD1 |
| 2 | CRC16 low bytes | 0x85 |
| 3 | CRC16 High Bytes | 0xEC |

Meter reply:

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xD1 |
| 2 | State bytes | 0x00 |
| 3 | CRC16 low bytes | CRCL |
| 4 | CRC16 High Bytes | CRCH |

Status bytes: 0x00--erase succeeded 0x01--erase failed

Send the upgrade data script 0xD2

The MCU upgrade file size is generally more than 40K, and the app needs to split the upgrade package and send it to the VCU in the following format.

The upgrade data is split into 128 bytes per pack.

App sending:

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xD2 |
| 2 | The package number is high bytes | 0x## |
| 3 | The package number is low byte | 0x## |
| 4 | The number of bytes of data is high | 0x00 |
| 5 | The number of bytes of data is low | 0x80 |
| 6 | Data 0 | 0x## |
| 7 | Data 1 | 0x## |
| 8 | Data 2 | 0x## |
| ... | ... | ... |
| ... | ... | ... |
| 132 | Data 126 | 0x## |
| 133 | Data 127 | 0x## |
| 134 | CRC16 low bytes | CRCL |
| 135 | CRC16 High Bytes | CRCH |

Meter reply:

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xD2 |
| 2 | The package number is high bytes | 0x## |
| 3 | The package number is low byte | 0x## |
| 4 | State bytes | 0x00 |
| 5 | CRC16 low bytes | CRCL |
| 6 | CRC16 High Bytes | CRCH |

The package number is the current package number received

State Bytes:

0x00--data received successfully  0x01-CRC error

## The upgrade is completed 0xD3 the script

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Baotou | 0XF0 |
| 1 | Script | 0xD3 |
| 2 | CRC16 low bytes | 0x04 |
| 3 | CRC16 High Bytes | 0x2D |

The command meter did not reply, and the instruction was sent once after all upgrade files were sent for 1 sent.

# 4.AT Instruction Service

AT服务  UUID：0000f2f0-0000-1000-8000-00805f9b34fb
	AT接收  UUID：0000f2f1-0000-1000-8000-00805f9b34fb（write）
	AT发送  UUID：0000f2f2-0000-1000-8000-00805f9b34fb（notify）

AT commands are used to set some parameters of Bluetooth devices, which are sent and received in ASCII code format

## Bluetooth Broadcast Name Query and Settings

| directive | Answer | Parameters |
| --- | --- | --- |
| Query: AT+NAME? | OK+NAME:Para | Para: Module name Up to 11 characters are allowed, including letters, numbers, underscores 默认 To=HW_ZAxx |
| 设置:AT+NAME[For] | OK+NAME:Para |  |

## Set/query encryption mode

| directive | Answer | Parameters |
| --- | --- | --- |
| Enquiry: AT+TYPE? | OK+TYPE:Para | Para: encryption mode A: No verification required B: Application-layer password encryption Default: A |
| 设置:AT+TYPE[For] | OK+TYPE:Para |  |

## Set/look up passwords

| directive | Answer | Parameters |
| --- | --- | --- |
| 验证密码:AT+PWD[For] | 验证成功:OK+PWD:Y 验证失败:OK+PWD:N | Para: Password6 digits Default: 888888 |
| 设置密码:AT+PWDM[For] | 回复 0K+PWDM:Para |  |

Example (without encryption):

Check the Bluetooth broadcast name:

The command format uses ASCII code to send and receive.

Write:  AT+NAME?

Notify:  OK+NAME:ZYD_0059950

Modify Bluetooth Broadcast Name:

The command format uses ASCII code to send and receive.

Write:  AT+NAME[AAABBBCCCDD]

Notify:  OK+NAME:AAABBBCCCDD

Successful modification:

# 5. VCU upgrade service

Upgrade the service UUID:f000ffc0-0451-4000-b000-000000000000
	Upgrade to receive UUID:f000ffc1-0451-4000-b000-000000000000
	Upgrade to send UUID:f000ffc2-0451-4000-b000-000000000000

Meter upgrades through the service

The upgrade file is a .bin file, where the first 16 bytes of the bin file are the upgrade firmware information, and the first 16 bytes are the upgrade data.

The following figure shows the first part of the bin file:

The first 16 bytes are parsed as follows (low bytes first):

Byte0-Byte3  upgrades the checksum bytes

byte4-byte5  待升级vcu版本号

Data 0x00 0x22 corresponding version number V2.2

Byte6-Byte7  upgrade file length in words (4 bytes).

Data 0xB4 0x22 Corresponding length 0x22B4

byte8-byte11 升级类型

Data 0x42 0x42 0x42 0x42  Upgrade Meter Program

Byte12-Byte13 Reserved

byte14-byte15 蓝牙协议栈版本

The data 0x12 0x00 corresponding version number V0.12

The app needs to check whether the version number of the instrument to be upgraded is different from the current version number of the instrument.

Instrument upgrade flow chart:

N

And

N

Y                                    N

## Read VCU firmware version information

App发送: UUID:f000ffc1-0451-4000-b000-00000000000000

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | directive | 0x00 |

VCU回复：  UUID：f000ffc1-0451-4000-b000-000000000000

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | VCU version number is 8 digits lower | 0x1A |
| 1 | The VCU version number is 8 digits higher | 0x00 |
| 2 | The firmware package length is low bytes | 0xB4 |
| 3 | The length of the firmware package is high bytes | 0x22 |
| 4 | Uid0 | 0x42 |
| 5 | Uid1 | 0x42 |
| 6 | Uid2 | 0x42 |
| 7 | Uid3 | 0x42 |
| 8 | The BLE stack version is 8 bits lower | 0x12 |
| 9 | The BLE stack version is 8 bits higher | 0x00 |

The app can only use the VCU version, and you don't need to pay attention to other data.

## Send the upgrade package information

The content sent is the first 16 bytes of the upgrade package

App发送: f000ffc1-0451-4000-b000-000000000000

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | Firmware package verification bytes | 0X6F |
| 1 | Firmware package verification bytes | 0x3C |
| 2 | Firmware package verification bytes | 0xD4 |
| 3 | Firmware package verification bytes | 0xA2 |
| 4 | VCU version number is 8 digits lower | 0x03 |
| 5 | The VCU version number is 8 digits higher | 0x01 |
| 6 | The firmware package length is low bytes | 0xB4 |
| 7 | The length of the firmware package is high bytes | 0x22 |
| 8 | Uid0 | 0x42 |
| 9 | Uid1 | 0x42 |
| 10 | Uid2 | 0x42 |
| 11 | Uid3 | 0x42 |
| 12 | Retained | 0xFF |
| 13 | Retained | 0xFF |
| 14 | The BLE stack version is 8 bits lower | 0x12 |
| 15 | The BLE stack version is 8 bits higher | 0x00 |

VCU回复：  f000ffc2-0451-4000-b000-000000000000

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | The package number is 8 digits lower | 0x00 |
| 1 | The package number is 8 digits higher | 0x00 |

It should be noted that the VCU reply is via FFC2

## Send the upgrade file

App发送: f000ffc2-0451-4000-b000-000000000000

The app needs to split the entire upgrade file into 16 bytes per package and send it to VCU, and the package number needs to be automatically added by 1, and the app can be sent at intervals of 10ms per package

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | The package number is 8 digits lower | 0x00 |
| 1 | The package number is 8 digits higher | 0x00 |
| 2-17 | Upgrade data 16 bytes |  |

Vcu回复：  f000ffc2-0451-4000-b000-000000000000

Only the VCU will receive an error in the data will reply The reply content is the package number, and the app needs to continue sending it down from the location of the received package number after receiving the reply

| Serial number | Byte definition | Example |
| --- | --- | --- |
| 0 | The package number is 8 digits lower | 0x## |
| 1 | The package number is 8 digits higher | 0x## |

Example (without encryption):

If the app in the box is in the wrong format, the VCU will reply to the package number and remind the app that it needs data with the package number 0x01 0x00.

# CRC

The CRC calculation range includes all bytes except CRC bytes, and the CRC calculation can directly call CalculateCRC16(), and the return value is the calculated CRC value.

const unsigned char CRCH[] =

{

0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,

0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,

0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,

0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,

0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,

0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,

0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,

0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,

0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,

0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,

0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,

0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,

0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,

0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,

0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,

0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,

0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,

0x40

};

const unsigned char CRCL[] =

{

0x00,0xC0,0xC1,0x01,0xC3,0x03,0x02,0xC2,0xC6,0x06,0x07,0xC7,0x05,0xC5,0xC4,

0x04,0xCC,0x0C,0x0D,0xCD,0x0F,0xCF,0xCE,0x0E,0x0A,0xCA,0xCB,0x0B,0xC9,0x09,

0x08,0xC8,0xD8,0x18,0x19,0xD9,0x1B,0xDB,0xDA,0x1A,0x1E,0xDE,0xDF,0x1F,0xDD,

0x1D,0x1C,0xDC,0x14,0xD4,0xD5,0x15,0xD7,0x17,0x16,0xD6,0xD2,0x12,0x13,0xD3,

0x11,0xD1,0xD0,0x10,0xF0,0x30,0x31,0xF1,0x33,0xF3,0xF2,0x32,0x36,0xF6,0xF7,

0x37,0xF5,0x35,0x34,0xF4,0x3C,0xFC,0xFD,0x3D,0xFF,0x3F,0x3E,0xFE,0xFA,0x3A,

0x3B,0xFB,0x39,0xF9,0xF8,0x38,0x28,0xE8,0xE9,0x29,0xEB,0x2B,0x2A,0xEA,0xEE,

0x2E,0x2F,0xEF,0x2D,0xED,0xEC,0x2C,0xE4,0x24,0x25,0xE5,0x27,0xE7,0xE6,0x26,

0x22,0xE2,0xE3,0x23,0xE1,0x21,0x20,0xE0,0xA0,0x60,0x61,0xA1,0x63,0xA3,0xA2,

0x62,0x66,0xA6,0xA7,0x67,0xA5,0x65,0x64,0xA4,0x6C,0xAC,0xAD,0x6D,0xAF,0x6F,

0x6E,0xAE,0xAA,0x6A,0x6B,0xAB,0x69,0xA9,0xA8,0x68,0x78,0xB8,0xB9,0x79,0xBB,

0x7B,0x7A,0xBA,0xBE,0x7E,0x7F,0xBF,0x7D,0xBD,0xBC,0x7C,0xB4,0x74,0x75,0xB5,

0x77,0xB7,0xB6,0x76,0x72,0xB2,0xB3,0x73,0xB1,0x71,0x70,0xB0,0x50,0x90,0x91,

0x51,0x93,0x53,0x52,0x92,0x96,0x56,0x57,0x97,0x55,0x95,0x94,0x54,0x9C,0x5C,

0x5D,0x9D,0x5F,0x9F,0x9E,0x5E,0x5A,0x9A,0x9B,0x5B,0x99,0x59,0x58,0x98,0x88,

0x48,0x49,0x89,0x4B,0x8B,0x8A,0x4A,0x4E,0x8E,0x8F,0x4F,0x8D,0x4D,0x4C,0x8C,

0x44,0x84,0x85,0x45,0x87,0x47,0x46,0x86,0x82,0x42,0x43,0x83,0x41,0x81,0x80,

0x40

};

/*************************************************************************

* *msgPtr needs to calculate the first address of the CRC data

* msgLen needs to calculate the length of the CRC data

* return calculated CRC value

*************************************************************************/

unsigned short int  CalculateCRC16(unsigned char  *msgPtr, unsigned short int msgLen)

{

unsigned char crcHigh = 0xFF; //high byte of CRC initialized

unsigned char crcLow = 0xFF; //low byte of CRC initialized

unsigned char index; //will index into CRC lookup table

while (msgLen--) //pass through message buffer

{

index = crcLow ^ (*(msgPtr++)); //calculate the CRC

crcLow = crcHigh ^ CRCH[index];

crcHigh = CRCL[index];

}

return (unsigned short int)((unsigned short int)(crcHigh<<8) | crcLow);

}

# Data encryption

## Encryption

We use encryption similar to single-table substitution, replacing 0-F with random numbers.

For example:

Key[16]={0x02,0x03,0x07,0x09,0x0A,0x01,0x04,0x08,0x0F,0x06,0x0D,0x0C,0x05,0x0B,0x00,0x0E};

0->2 1->3 2->7 3->9 4->A 5->1 6->4 7->8 8->F 9->6 A->D B->C C->5 D->B E->0 F->E

明文：tab[3] = {0x0F,0x12,0xE3}

密文：tab[3] = {0x2E,0x37,0x09}

For enhanced encryption, we employ 16 sets of keys

unsigned char const Key[16][16]={

{0x02,0x03,0x07,0x09,0x0A,0x01,0x04,0x08,0x0F,0x06,0x0D,0x0C,0x05,0x0B,0x00,0x0E},//0

{0x01,0x06,0x0A,0x05,0x07,0x02,0x0E,0x00,0x0C,0x03,0x0B,0x0F,0x09,0x0D,0x08,0x04},//1

{0x0F,0x03,0x09,0x07,0x0E,0x04,0x01,0x08,0x02,0x0D,0x06,0x00,0x05,0x0B,0x0C,0x0A},//2

{0x01,0x06,0x0F,0x09,0x0A,0x02,0x04,0x0D,0x07,0x03,0x08,0x00,0x05,0x0B,0x0C,0x0E},//3

{0x00,0x03,0x07,0x0E,0x08,0x01,0x0B,0x0A,0x0F,0x06,0x0D,0x0C,0x05,0x04,0x02,0x09},//4

{0x0F,0x00,0x0A,0x02,0x07,0x03,0x04,0x08,0x09,0x06,0x0C,0x0D,0x05,0x0B,0x01,0x0E},//5

{0x03,0x02,0x09,0x07,0x01,0x0A,0x0F,0x06,0x04,0x08,0x00,0x0C,0x0E,0x0B,0x0D,0x05},//6

{0x07,0x03,0x02,0x09,0x0F,0x0D,0x0A,0x08,0x0B,0x06,0x01,0x0C,0x05,0x04,0x00,0x0E},//7

{0x02,0x03,0x0B,0x01,0x0A,0x09,0x05,0x08,0x0C,0x07,0x0D,0x0F,0x04,0x06,0x0E,0x00},//8

{0x01,0x03,0x04,0x09,0x05,0x02,0x07,0x00,0x0E,0x06,0x0D,0x08,0x0A,0x0B,0x0C,0x0F},//9

{0x00,0x07,0x0D,0x0B,0x0A,0x06,0x04,0x08,0x0F,0x01,0x03,0x0C,0x05,0x0E,0x02,0x09},//A

{0x0F,0x03,0x0E,0x0B,0x06,0x08,0x04,0x01,0x02,0x0C,0x0D,0x0A,0x05,0x00,0x09,0x07},//B

{0x08,0x0C,0x07,0x0E,0x0A,0x01,0x06,0x02,0x0F,0x04,0x0D,0x03,0x00,0x0B,0x05,0x09},//C

{0x02,0x0E,0x0A,0x07,0x09,0x06,0x04,0x08,0x00,0x01,0x05,0x0C,0x0D,0x0B,0x0F,0x03},//D

{0x0E,0x00,0x0B,0x05,0x0C,0x0D,0x0F,0x06,0x04,0x08,0x01,0x0A,0x09,0x07,0x03,0x02},//E

{0x01,0x0F,0x03,0x07,0x06,0x00,0x0D,0x0C,0x02,0x09,0x0E,0x05,0x08,0x0A,0x0B,0x04},//F

};

Select the key by accumulating and lowering the four digits by Serial Number

For example, SN is "0123456789123"

秘钥为 Key[keytype][16]

Sum = 0x30+0x31+0x32+0x33+0x34+0x35+0x36+0x37+0x38+0x39+0x31+0x32+0x33

Sum = 0x02A3

keytype= Sum & 0x000F                 keytype= 0x0003

Ki[keytipe][16] = ki[3][16] =

{0x01,0x06,0x0F,0x09,0x0A,0x02,0x04,0x0D,0x07,0x03,0x08,0x00,0x05,0x0B,0x0C,0x0E},//3

/*Encrypted*/

/**********************************************************************************

*parameter *tabIn  The data header address (plaintext) that needs to be encrypted

*Parameter  *tabOutThe first address (ciphertext) of the encrypted data is stored

*Parameter len The length of the data that needs to be encrypted

*Parameter type key selection

**********************************************************************************/

void encrypt(unsigned char *tabIn,unsigned char *tabOut,unsigned char len,unsigned char type)

{

unsigned char i=0;

unsigned char codeL,codeH;

for(i=0; i<len; i++)

{

codeL = tabIn[i]&0x0F;

codeH = (tabIn[i]&0xF0)>>4;

tabOut[i] = 0;

tabOut[i] |= key[type][codeL];

tabOut[i] |= (key[type][codeH]<<4);

}

}

/*decrypt*/

/**********************************************************************************

*parameter *tabIn  The data header address (ciphertext) that needs to be decrypted

*parameter *tabOut: The first address of the decrypted data (plaintext).

*The parameter len requires the length of the data to be decrypted

*Parameter type key selection

**********************************************************************************/

void decrypt(unsigned char *tabIn,unsigned char *tabOut,unsigned char len,unsigned char type)

{

unsigned char i=0,j=0;

unsigned char codeL,codeH;

for(i=0; i<len; i++)

{

codeL = tabIn[i]&0x0F;

codeH = (tabIn[i]&0xF0)>>4;

tabOut[i] = 0;

for(j=0; j<16; j++)

{

if(codeL==key[type][j])

{

tabOut[i] |= j;

}

if(codeH==key[type][j])

{

tabOut[i] |=(j<<4);

}

}

}

}

Example:

Send the mobile phone to send the command code to read the skateboard information:

Before encryption: F0 B0 05 C4 30

Encryption is used with the key [3][16]

After encryption: E1 01 12 5A 91

## Encrypted byte description

- Device Information的Serial Number 不用加密

- When VCU is upgraded, the package number of the upgrade data is not encrypted

- When the VCU is upgraded, the number of bytes sent or replied to is less than 3 bytes is not encrypted

In addition to the above points, all other data needs to be encrypted.