Bluetooth communication protocol

Applicable instrument: HW8076

directory

Instruction set 4

1. Bluetooth service UUID5

2.Device Information Service 6

3. Transparent transmission service7

The app reads the scooter running data instruction code 0xA08

The app reads scooter BMS data instruction code 0xA19

The phone sends instruction code 0xB011 to read skateboard information

The APP issues control data instruction code 0xC012

Controller MCU upgrade 13

Request upgrade instruction code 0xD015

Erase MCU flash code 0xD1 15

Upgrade completion command code 0xD316

4.AT instruction service 17

Bluetooth broadcast name query with setting 17

Set/query encryption mode 18

Set/query password 18

5. VCU upgrade service 20

Read VCU firmware version information 22

Send upgrade package information 23

Send the upgrade file 23

CRC24

Data encryption26

Encryption method 26

Cryptographic bytes description 29

Modify records

| Communication protocol version | date | Modify the content |
| --- | --- | --- |
| V1.0 | 2022-01-17 | First release |
| V1.1 | 2022-02-25 | Add VCU upgrades, add AT commands related to password change, and modify SN code acquisition methods |
| V1.2 | 2022-09-13 | Modify cruise speed limits |
| V1.3 | 2022-10-13 | Revise page 11 of the instrumentation software version number and page 21 of BYTE4, BYTE5 software version number |
| V.4 | 2023-01-30 | Modify the MCU upgrade protocol |

Glossary

| Acronym | Definition |
| --- | --- |
| BMS | Battery Management System |
| MCU | Motor Controller Uint |
| RPM | Revolution Per Minute |
| SN | Serial Number |
| VCU | Vehicle Control Unit |
| CMD | Command |

Instruction set

| Instruction code | illustrate | remark |
| --- | --- | --- |
| 0xA0 | Read scooter running data instructions | The app does not read the meter and it will be uploaded automatically |
| 0xA1 | Read battery data instructions | The app does not read the meter and it will be uploaded automatically |

| Instruction code | illustrate | remark |
| --- | --- | --- |
| 0xB0 | Read version information and model instructions | Gauges are not automatically uploaded |

| Instruction code | illustrate | remark |
| --- | --- | --- |
| 0xC0 | APP writes instructions | Set the function switch signal, speed limit value, etc |

| Instruction code | illustrate | remark |
| --- | --- | --- |
| 0xD0 | Query whether the controller allows upgrade |  |
| 0xD1 | Erase controller flash instructions |  |
| 0xD2 | Upgrade data |  |
| 0xD3 | The upgrade is complete |  |

1. Bluetooth service UUID

The picture above is all the service channels of our Bluetooth chip, and in this project we need to use 3 of these services in addition to Device Information, namely:

UUID：0000fff0-0000-1000-8000-00805f9b34fb

UUID：0000f2f0-0000-1000-8000-00805f9b34fb

UUID：f000ffc0-0451-4000-b000-000000000000

The communication framework diagram is as follows:

2. Device Information Service

The SN code is obtained through Device Information's Serial Number String

The SN code is in ASCII format and is 13 characters

3. Transparent transmission service

Transparent transmission service UUID: 000 0fff0-0000-1000-8000-00805f9b34fb transparent transmission receiving UUID:
	 0000fff1-0000-1000-8000-00805f9b34fb  (write) 
	Transparent transmission UUID: 0000fff2-00 00-1000-8000-00805f9b34fb (notify ）

The app can obtain the operating data and BMS data of the scooter through this service, as well as control the status of the scooter and upgrade the controller.

The framework diagram is as follows:

The app reads the scooter running data instruction code 0xA0

The app can read the data without sending this command, because the meter will actively upload this frame of data on a regular basis.

Example:

Write:	F0 A0 05 C9 F0

Notify:	F0 A0 19 00 00 00 00 19 10 19 0A 00 00 10 00 C8 19 19 00 C8 00 00 00 03 4F

The resolution of each byte is as follows:

App send:

| serial number | Byte definition |  | Sample |
| --- | --- | --- | --- |
| 0 | turban |  | 0XF0 |
| 1 | Instruction code |  | 0xA0 |
| 2 | Total frame bytes |  | 0x05 |
| 3 | CRC16 low byte |  | 0xC9 |
| 4 | CRC16 high bytes |  | 0xF0 |

Instrument reply: (This package meter will be actively uploaded to the APP at 200ms).

| serial number | Byte definition | Sample | Content definition | remark |
| --- | --- | --- | --- | --- |
| 0 | turban | 0XF0 |  |  |
| 1 | Instruction code | 0xA0 |  |  |
| 2 | Total frame bytes | 0x19 |  | 0x19=25 |
| 3 | Failure code high bytes | 0x00 |  | See definitions below |
| 4 | Failure code low bytes | 0x00 |  | See definitions below |
| 5 | Control function high bytes | 0x00 |  | See definitions below |
| 6 | Control function low bytes | 0x00 |  | See definitions below |
| 7 | Cruise speed setting | 0x19 | 5-25km/h can enter the fixed speed cycle | 0x19=25km/h |
| 8 | Current speed | 0x10 |  | 0x10=16km/h |
| 9 | Maximum speed limit | 0x19 | Maximum speed 25km/h | 0x19=25km/h |
| 10 | Subtotal mileage | 0x0A | Mileage of this ride | 0x0A=10km |
| 11 | Total mileage in high bytes | 0x00 |  | Km |
| 12 | Total mileage low bytes | 0x00 |  | Km |
| 13 | Remaining range | 0x10 | Estimated based on the amount of electricity | 0x10=16km |
| 14 | The current limit value is high bytes | 0x00 | Unit 0.1A | 0x00C8=200 means 20A |
| 15 | The current limit value is low bytes | 0xC8 |  |  |
| 16 | Current motor temperature | 0x19 | Range 0-255°C | 25℃ |
| 17 | Current controller temperature | 0x19 | Range 0-255°C | 25℃ |
| 18 | The current motor speed is high byte | 0x00 | Unit RPM | 200RPM |
| 19 | The current motor speed is low byte | 0xC8 |  |  |
| 20 | retain | 0x00 |  |  |
| 21 | retain | 0x00 |  |  |
| 22 | retain | 0x00 |  |  |
| 23 | CRC16 low byte | CRCL |  | MODBUS——CRC16 |
| 24 | CRC16 high bytes | CRCH |  |  |

Control function byte definitions:

| Bit7-Bit6 | Bit5 | Bit4 | Bit3-Bit2 | Bit1-Bit0 |
| --- | --- | --- | --- | --- |
| Turn signals: 0x0: off 0x1: turn right 0x2: Turn left | Cruise control: 0: Close 1: Open | Headlights: 0: Close 1: Open | retain | Gear: 0x0: 1st gear 0x1: 2nd gear 0x2: 3 gears |

| Bit15-Bit11 | Bit10 | Bit9 | Bit8 |
| --- | --- | --- | --- |
| retain | Start mode: 0: Glide start 1: Zero start | Unit: 0:Km 1:Mile | Lock status: 0: unlock 1: locked |

Fault code byte definition:

| Fault code | illustrate |
| --- | --- |
| Bit0 |  |
| Bit1 | 0: Normal 1: Brake finger fault |
| Bit2 | 0: Normal 1: Throttle finger fault |
| Bit3 | 0: Normal 1: Instrument and controller communication failure |
| Bit4 | 0: Normal 1: Controller overcurrent |
| Bit5 | retain |
| Bit6 | retainfb |
| Bit7 | 0: Normal 1: Hall failure |
| Bit8 | retain |
| Bit9 | 0: Normal 1: Op amp bias fault |
| Bit10-bit15 | To be defined |

E0: Motor Temp Out of Range
E1: Brake Fault
E2: Throttle Fault
E3: Communication Fault
E4 Overcurrent Fault
E5: Undervoltage Fault
E6: Overvoltage Fault

E7: Motor Hall Fault
E8: Motor Phase Fault
E9: MCU Over Temperature
E10: MCU Temp Out of Range
E11: Motor Over Temperature
E12: Bluetooth Fault
E13: Battery Fault

The app reads the scooter BMS data instruction code 0xA1

The app can read the data without sending this command, because the meter will actively upload this frame of data on a regular basis.

Example:

Write:	F0 A1 05 C8 60

Notify:	F0 A1 19 00 00 00 0A 01 A4 64 00 10 0F A0 0F A0 19 00 00 00 00 00 00 A2 C0

The resolution of each byte is as follows:

App send:

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xA1 |
| 2 | Total frame bytes | 0x05 |
| 3 | CRC16 low byte | 0xC8 |
| 4 | CRC16 high bytes | 0x60 |

Instrument reply: (This package meter will be actively uploaded to the APP at 200ms).

| serial number | Byte definition | Sample | Content definition | remark |
| --- | --- | --- | --- | --- |
| 0 | turban | 0XF0 |  |  |
| 1 | Instruction code | 0xA1 |  |  |
| 2 | Total frame bytes | 0x19 |  |  |
| 3 | Battery status high bytes | 0x00 |  | See definitions below |
| 4 | Battery status low bytes | 0x00 |  |  |
| 5 | Battery current high bytes | 0x00 | Signed unit 0.1A | 0x000A = 10 1.0A |
| 6 | Battery current low bytes | 0x0A |  |  |
| 7 | The current battery voltage is high bytes | 0x01 | Unit 0.1V | 0x01A4=420 42.0V |
| 8 | Battery current voltage low bytes | 0xA4 |  |  |
| 9 | Current battery level | 0x64 | Unit 1%. | 100% |
| 10 | The number of charge cycles is high byte | 0x00 |  | 0x0010 = 16 times |
| 11 | The number of charge cycles is low byte | 0x10 |  |  |
| 12 | Battery capacity high bytes | 0x0F | Unit mAh |  |
| 13 | Battery capacity low bytes | 0xA0 |  |  |
| 14 | High bytes of capacity remaining | 0x0F | Unit mAh |  |
| 15 | Low bytes of capacity remaining | 0xA0 |  |  |
| 16 | Battery temperature | 0x19 | Signed units °C | 0x19 = 25 25℃ |
| 17 | retain | 0x00 |  |  |
| 18 | retain | 0x00 |  |  |
| 19 | retain | 0x00 |  |  |
| 20 | retain | 0x00 |  |  |
| 21 | retain | 0x00 |  |  |
| 22 | retain | 0x00 |  |  |
| 23 | CRC16 low byte | CRCL |  | MODBUS——CRC16 |
| 24 | CRC16 high bytes | CRCH |  |  |

Note: The instruction 0xA# packet app can read without sending instructions The  instrument will automatically poll and upload these data

Battery status byte definition:

| bit15-bit1 | bit0 |
| --- | --- |
| retain | Charging sign: 1 - charged 0 - not charged |
| Bit32-bit16 |  |
| retain |  |

The phone sends instruction code 0xB0 to read skateboard information

Example:

Write:	F0 B0 05 C4 30

Notify:	F0 B0 20 00 01 30 30 30 30 30 30 30 30 30 30 30 30 30 00 00 00 10 10 20 20 30 30 00 		00 00 F6 9E

The resolution of each byte is as follows:

App send:

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xB0 |
| 2 | Total frame bytes | 0x05 |
| 3 | CRC16 low byte | 0xC4 |
| 4 | CRC16 high bytes | 0x30 |

Meter reply:

| serial number | Byte definition | Sample | Content definition | remark |
| --- | --- | --- | --- | --- |
| 0 | turban | 0XF0 |  |  |
| 1 | Instruction code | 0xB0 |  |  |
| 2 | Total frame bytes | 0x19 |  | 0x19=25 |
| 3 | Model high byte | 0x00 | Model 0x0001=S008 Base |  |
| 4 | Model low byte | 0x01 |  |  |
| 5 | Meter hardware version | 0x10 | Indicates version 1.0 | V1.0 |
| 6 | Meter software version | 0x22 | Indicates version 2.2 | V2.2 |
| 7 | Controller hardware version | 0x20 | Indicates version 2.0 | V2.0 |
| 8 | Controller software version | 0x20 | Indicates version 2.0 | V2.0 |
| 9 | BMS hardware version | 0x30 | Indicates version 3.0 | V3.0 |
| 10 | BMS software version | 0x30 | Indicates version 3.0 | V3.0 |
| 11 | retain | 0x00 |  |  |
| 12 | retain | 0x00 |  |  |
| 13 | retain | 0x00 |  |  |
| 14 | retain | 0x00 |  |  |
| 15 | retain | 0x00 |  |  |
| 16 | retain | 0x00 |  |  |
| 17 | retain | 0x00 |  |  |
| 18 | retain | 0x00 |  |  |
| 19 | retain | 0x00 |  |  |
| 20 | retain | 0x00 |  |  |
| 21 | retain | 0x00 |  |  |
| 22 | retain | 0x00 |  |  |
| 23 | CRC16 low byte | CRCL |  |  |
| 24 | CRC16 high bytes | CRCH |  |  |

The APP issues control data instruction code 0xC0

The App can be used to control scooter gear, headlight switch, cruise control switch, lock status, unit, start mode, speed limit value, etc. through this command.

Example:

Write:	F0 C0 0F 00 00 19 19 00 00 00 00 00 00 39 E8

Notify:	F0 C0 06 00 30 B8

The resolution of each byte is as follows:

App send:

| serial number | Byte definition | Sample | Content definition | remark |
| --- | --- | --- | --- | --- |
| 0 | turban | 0XF0 |  |  |
| 1 | Instruction code | 0xC0 |  |  |
| 2 | Total frame bytes | 0x0F |  | 0x0A=10 |
| 3 | Control function high bytes | 0x00 |  |  |
| 4 | Control function low bytes | 0x00 |  |  |
| 5 | Cruise speed setting | 0x05 | Speed>=5km/h can enter the fixed speed cycle | 0x05=5kph |
| 6 | Maximum speed limit | 0x19 | Maximum speed 25km/h | 0x19=25km/h |
| 7 | retain | 0x00 |  |  |
| 8 | retain | 0x00 |  |  |
| 9 | retain | 0x00 |  |  |
| 10 | retain | 0x00 |  |  |
| 11 | retain | 0x00 |  |  |
| 12 | retain | 0x00 |  |  |
| 13 | CRC16 low byte | CRCL |  | MODBUS——CRC16 |
| 14 | CRC16 high bytes | CRCH |  |  |

Data Received Success Meter Reply:

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xC0 |
| 2 | Total frame bytes | 0x06 |
| 3 | Status bytes | 0x00 |
| 4 | CRC16 low byte | CRCL |
| 5 | CRC16 high bytes | CRCH |

Status byte: 0x00--Data received successfully 0x01--CRC check error

Control function byte definitions:

| Bit7-Bit6 | Bit5 | Bit4 | Bit3-Bit2 | Bit1-Bit0 |
| --- | --- | --- | --- | --- |
|  | Cruise control: 0: Close 1: Open | Headlights: 0: Close 1: Open | retain | Gear: 0x0: 1st gear 0x1: 2nd gear 0x2: 3 gears |

| Bit15-Bit11 | Bit10 | Bit9 | Bit8 |
| --- | --- | --- | --- |
| retain | Start mode: 0: Glide start 1: Zero start | Unit: 0:Km 1:Mile | Lock status: 0: unlock 1: locked |

Controller MCU upgrade

When the MCU is upgraded, the app first sends the upgrade file to the VCU, and then the VCU is transferred to the MCU, and if the instruction is not replied in 1s during the upgrade process, it will be resent, and the number of times sent will be exceeded 5 times is considered to have failed the upgrade

MCU upgrade flowchart:

N

And

N

And

N

N

and

N

Y									   N

The N

And

And

Before upgrading, you need to compare the hardware version of the MCU with the hardware version of the upgrade file, and the hardware version can be upgraded only if the hardware version is the same;

The maximum number of retransmissions is 5 times, and the interval is 1s;

Request upgrade code 0xD0

App send:

| serial number | Byte definition |  |
| --- | --- | --- |
| 0 | turban | 0xF0 |
| 1 | Instruction code | 0xD0 |
| 2 | CRC16 low byte | 0x44 |
| 3 | CRC16 high bytes | 0x2C |

Meter reply:

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xD0 |
| 2 | Status bytes | 0x00 |
| 3 | CRC16 low byte | CRCL |
| 4 | CRC16 high bytes | CRCH |

Status bytes: 0x00 -- allow upgrade  0x01 - no upgrade allowed

Erase the MCU flash codex 0xD1

App send:

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xD1 |
| 2 | CRC16 low byte | 0x85 |
| 3 | CRC16 high bytes | 0xEC |

Meter reply:

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xD1 |
| 2 | Status bytes | 0x00 |
| 3 | CRC16 low byte | CRCL |
| 4 | CRC16 high bytes | CRCH |

Status bytes: 0x00 -- erase successful 0x01 - erase failed

Send upgrade data instruction code 0xD2

The MCU upgrade file size is generally more than 40K, and the app needs to split the upgrade package and send it to the VCU in the following format.

Upgrade data is split into 128 bytes per packet.

App send:

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xD2 |
| 2 | The packet number is high byte | 0x## |
| 3 | Packet number low byte | 0x## |
| 4 | The number of data is high bytes | 0x00 |
| 5 | Low number of bytes of data | 0x80 |
| 6 | Data 0 | 0x## |
| 7 | Data 1 | 0x## |
| 8 | Data 2 | 0x## |
| ... | ... | ... |
| ... | ... | ... |
| 132 | Data 126 | 0x## |
| 133 | Data 127 | 0x## |
| 134 | CRC16 low byte | CRCL |
| 135 | CRC16 high bytes | CRCH |

Meter reply:

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xD2 |
| 2 | The packet number is high byte | 0x## |
| 3 | Packet number low byte | 0x## |
| 4 | Status bytes | 0x00 |
| 5 | CRC16 low byte | CRCL |
| 6 | CRC16 high bytes | CRCH |

The packet number is the current packet number received

Status byte:

0x00 -- Data received successfully 0x01 - CRC error

Upgrade completion instruction code 0xD3

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | turban | 0XF0 |
| 1 | Instruction code | 0xD3 |
| 2 | CRC16 low byte | 0x04 |
| 3 | CRC16 high bytes | 0x2D |

The command meter did not reply, and the command was sent once after 1s after all the upgrade files were sent.

4.AT instruction service

AT服务  UUID：0000f2f0-0000-1000-8000-00805f9b34fb
	AT接收  UUID：0000f2f1-0000-1000-8000-00805f9b34fb（write）
	AT发送  UUID：0000f2f2-0000-1000-8000-00805f9b34fb（notify）

The AT command is used to set some parameters of the Bluetooth device, which is sent and received in ASCII code format

## Bluetooth broadcast name query and setting

| directives | Response | parameter |
| --- | --- | --- |
| Query: AT+NAME? | OK+NAME:Para | Para: The module name The maximum is 11 characters, including letters, numbers, underscores The default para=HW_ZAxx |
| Settings: AT+NAME[Para]. | OK+NAME:Para |  |

Set/query encryption mode

| directives | Response | parameter |
| --- | --- | --- |
| Query: AT+TYPE? | OK+TYPE:Para | Para: Encryption mode A: No verification required B: Application layer password encryption Default value: A |
| Settings: AT+TYPE[Para]. | OK+TYPE:Para |  |

Set/query password

| directives | Response | parameter |
| --- | --- | --- |
| Authentication password: AT+PWD[Para]. | Authentication successful: OK+PWD:Y Authentication failed: OK+PWD:N | Para: Password 6 digits Default: 888888 |
| Set the password: AT+PWDM [Para]. | Reply to 0K+PWDM:Para |  |

Example (no encryption):

Query the Bluetooth broadcast name:

The instruction format is sent and received using ASCII code.

Write:  AT+NAME?

Notify:  OK+NAME:ZYD_0059950

To modify the Bluetooth broadcast name:

The instruction format is sent and received using ASCII code.

Write:  AT+NAME[AAABBBCCCDD]

Notify:  OK+NAME:AAABBBCCCDD

Modified successfully:

5. VCU upgrade service

Upgrade service UUID: f0
	 00ffc0-0451-4000-b000-00000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000 
	 Upgrade UUID: f000ffc2-0451-4000-b000-00000000000000

Instrument upgrades with this service

The upgrade file is a .bin file, where the first 16 bytes of the bin file are the upgrade firmware information, and the 16 bytes after the upgrade data.

The following figure is the previous part of the bin file:

The first 16 bytes are parsed as follows (low bytes first):

Byte0-Byte3  upgrade data check byte

BYTE4-BYTE5 VCU version number to be upgraded

The data 0x00 0x22 corresponds to version number V2.2

Byte6-Byte7 upgrade file length, unit: word (4 bytes).

The data 0xB4 0x22 corresponds to the length 0x22B4

Byte8-Byte11 upgrade type

Data 0x42 0x42 0x42 0x42  Upgrade meter program

Byte12-Byte13 reserved

Byte14 - Byte15 Bluetooth protocol stack version

The data 0x12 0x00 corresponds to version number V0.12

The app needs to detect whether the version number of the instrument to be upgraded is different from the current version number of the instrument, and if the version number is the same, the meter will not be upgraded.

Instrument upgrade flow chart:

N

Y

N

The N

Read the VCU firmware version information

App sending:  UUID:f000ffc1-0451-4000-b000-0000000000000

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | directives | 0x00 |

VCU Re:  UUID:f000ffc1-0451-4000-b000-0000000000000

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | The VCU version number is 8 bits lower | 0x1A |
| 1 | The VCU version number is 8 bits high | 0x00 |
| 2 | The firmware package length is low bytes | 0xB4 |
| 3 | The firmware package length is high bytes | 0x22 |
| 4 | Uid0 | 0x42 |
| 5 | Uid1 | 0x42 |
| 6 | Uid2 | 0x42 |
| 7 | Uid3 | 0x42 |
| 8 | The BLE protocol stack version is 8 bits lower | 0x12 |
| 9 | The BLE stack version is 8 bits higher | 0x00 |

The app can only use the VCU version, and other data can be ignored.

Send the upgrade package information

The content sent is the first 16 bytes of the upgrade package

App sending: f000ffc1-0451-4000-b000-0000000000000

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | Firmware package check byte | 0X6F |
| 1 | Firmware package check byte | 0x3C |
| 2 | Firmware package check byte | 0xD4 |
| 3 | Firmware package check byte | 0xA2 |
| 4 | The VCU version number is 8 bits lower | 0x03 |
| 5 | The VCU version number is 8 bits high | 0x01 |
| 6 | The firmware package length is low bytes | 0xB4 |
| 7 | The firmware package length is high bytes | 0x22 |
| 8 | Uid0 | 0x42 |
| 9 | Uid1 | 0x42 |
| 10 | Uid2 | 0x42 |
| 11 | Uid3 | 0x42 |
| 12 | retain | 0xFF |
| 13 | retain | 0xFF |
| 14 | The BLE protocol stack version is 8 bits lower | 0x12 |
| 15 | The BLE stack version is 8 bits higher | 0x00 |

VCU Re:  f000ffc2-0451-4000-b000-0000000000000

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | The packet number is 8 bits lower | 0x00 |
| 1 | The package number is 8 bits high | 0x00 |

It should be noted that the VCU reply is through FFC2

Send the upgrade file

App sending: f000ffc2-0451-4000-b000-0000000000000

The app needs to split the entire upgrade file into 16 bytes per packet and send it to the VCU, and the packet number needs to be automatically added by 1, and the app can send it every packet interval of 10ms

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | The packet number is 8 bits lower | 0x00 |
| 1 | The package number is 8 bits high | 0x00 |
| 2-17 | Upgrade data 16 bytes |  |

VCU Re:  f000ffc2-0451-4000-b000-00000000000000

Only VCU will reply if the data is wrong The  reply content is the package number, and the app needs to continue to send from the received packet number after receiving the reply

| serial number | Byte definition | Sample |
| --- | --- | --- |
| 0 | The packet number is 8 bits lower | 0x## |
| 1 | The package number is 8 bits high | 0x## |

Example (no encryption):

If the app in the box sends the wrong format, the  VCU will reply with the package number, reminding the app that it needs data with a 0x01 0x00 package number.

CRC

The CRC calculation range includes all bytes except the CRC byte, and the CRC calculation can directly call CalculateCRC16(), and the return value is the calculated CRC value.

const unsigned char CRCH[] =

{

0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,

0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,

0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,

0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,

0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,

0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,

0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,

0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,

0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,

0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,

0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,

0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,

0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,

0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x01,0xC0,

0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,

0xC0,0x80,0x41,0x00,0xC1,0x81,0x40,0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,

0x00,0xC1,0x81,0x40,0x01,0xC0,0x80,0x41,0x01,0xC0,0x80,0x41,0x00,0xC1,0x81,

0x40

};

const unsigned char CRCL[] =

{

0x00,0xC0,0xC1,0x01,0xC3,0x03,0x02,0xC2,0xC6,0x06,0x07,0xC7,0x05,0xC5,0xC4,

0x04,0xCC,0x0C,0x0D,0xCD,0x0F,0xCF,0xCE,0x0E,0x0A,0xCA,0xCB,0x0B,0xC9,0x09,

0x08,0xC8,0xD8,0x18,0x19,0xD9,0x1B,0xDB,0xDA,0x1A,0x1E,0xDE,0xDF,0x1F,0xDD,

0x1D,0x1C,0xDC,0x14,0xD4,0xD5,0x15,0xD7,0x17,0x16,0xD6,0xD2,0x12,0x13,0xD3,

0x11,0xD1,0xD0,0x10,0xF0,0x30,0x31,0xF1,0x33,0xF3,0xF2,0x32,0x36,0xF6,0xF7,

0x37,0xF5,0x35,0x34,0xF4,0x3C,0xFC,0xFD,0x3D,0xFF,0x3F,0x3E,0xFE,0xFA,0x3A,

0x3B,0xFB,0x39,0xF9,0xF8,0x38,0x28,0xE8,0xE9,0x29,0xEB,0x2B,0x2A,0xEA,0xEE,

0x2E,0x2F,0xEF,0x2D,0xED,0xEC,0x2C,0xE4,0x24,0x25,0xE5,0x27,0xE7,0xE6,0x26,

0x22,0xE2,0xE3,0x23,0xE1,0x21,0x20,0xE0,0xA0,0x60,0x61,0xA1,0x63,0xA3,0xA2,

0x62.0x66.0xA6.0xA7.0x67.0xA5.0x65.0x64.0xA4.0x6C.0xAC.0xAD.0x6D.0xAF.0x6F,

0x6E,0xAE,0xAA,0x6A,0x6B,0xAB,0x69,0xA9,0xA8,0x68,0x78,0xB8,0xB9,0x79,0xBB,

0x7B,0x7A,0xBA,0xBE,0x7E,0x7F,0xBF,0x7D,0xBD,0xBC,0x7C,0xB4,0x74,0x75,0xB5,

0x77,0xB7,0xB6,0x76,0x72,0xB2,0xB3,0x73,0xB1,0x71,0x70,0xB0,0x50,0x90,0x91,

0x51,0x93,0x53,0x52,0x92,0x96,0x56,0x57,0x97,0x55,0x95,0x94,0x54,0x9C,0x5C,

0x5D,0x9D,0x5F,0x9F,0x9E,0x5E,0x5A,0x9A,0x9B,0x5B,0x99,0x59,0x58,0x98,0x88,

0x48,0x49,0x89,0x4B,0x8B,0x8A,0x4A,0x4E,0x8E,0x8F,0x4F,0x8D,0x4D,0x4C,0x8C,

0x44,0x84,0x85,0x45,0x87,0x47,0x46,0x86,0x82,0x42,0x43,0x83,0x41,0x81,0x80,

0x40

};

/*************************************************************************

* *msgPtr needs to calculate the first address of the CRC data

* msgLen needs to calculate the length of the CRC data

* return Calculated CRC value

*************************************************************************/

unsigned short int  CalculateCRC16(unsigned char  *msgPtr, unsigned short int msgLen)

{

unsigned char crcHigh = 0xFF; //high byte of CRC initialized

unsigned char crcLow = 0xFF; //low byte of CRC initialized

unsigned char index; //will index into CRC lookup table

while (msgLen--) //pass through message buffer

{

index = crcLow ^ (*(msgPtr++)); //calculate the CRC

crcLow = crcHigh ^ CRCH[index];

crcHigh = CRCL[index];

}

return (unsigned short int)((unsigned short int)(crcHigh<<8) | crcLow);

}

Data encryption

Encryption method

We use encryption similar to single-table substitution, replacing 0-F with random numbers.

For example:

Key[16]={0x02,0x03,0x07,0x09,0x0A,0x01,0x04,0x08,0x0F,0x06,0x0D,0x0C,0x05,0x0B,0x00,0x0E};

0->2  1->3  2->7  3->9  4->A  5->1  6->4  7->8  8->F  9->6  A->D  B->C  C->5  D->B  E->0  F->E

Plaintext: tab[3] = {0x0F,0x12,0xE3}

ciphertext: tab[3] = {0x2E,0x37,0x09}

To enhance encryption, we use 16 sets of keys

unsigned char const Key[16][16]={

{0x02,0x03,0x07,0x09,0x0A,0x01,0x04,0x08,0x0F,0x06,0x0D,0x0C,0x05,0x0B,0x00,0x0E},//0

{0x01,0x06,0x0A,0x05,0x07,0x02,0x0E,0x00,0x0C,0x03,0x0B,0x0F,0x09,0x0D,0x08,0x04},//1

{0x0F,0x03,0x09,0x07,0x0E,0x04,0x01,0x08,0x02,0x0D,0x06,0x00,0x05,0x0B,0x0C,0x0A},//2

{0x01,0x06,0x0F,0x09,0x0A,0x02,0x04,0x0D,0x07,0x03,0x08,0x00,0x05,0x0B,0x0C,0x0E},//3

{0x00,0x03,0x07,0x0E,0x08,0x01,0x0B,0x0A,0x0F,0x06,0x0D,0x0C,0x05,0x04,0x02,0x09},//4

{0x0F,0x00,0x0A,0x02,0x07,0x03,0x04,0x08,0x09,0x06,0x0C,0x0D,0x05,0x0B,0x01,0x0E},//5

{0x03,0x02,0x09,0x07,0x01,0x0A,0x0F,0x06,0x04,0x08,0x00,0x0C,0x0E,0x0B,0x0D,0x05},//6

{0x07,0x03,0x02,0x09,0x0F,0x0D,0x0A,0x08,0x0B,0x06,0x01,0x0C,0x05,0x04,0x00,0x0E},//7

{0x02,0x03,0x0B,0x01,0x0A,0x09,0x05,0x08,0x0C,0x07,0x0D,0x0F,0x04,0x06,0x0E,0x00},//8

{0x01,0x03,0x04,0x09,0x05,0x02,0x07,0x00,0x0E,0x06,0x0D,0x08,0x0A,0x0B,0x0C,0x0F},//9

{0x00,0x07,0x0D,0x0B,0x0A,0x06,0x04,0x08,0x0F,0x01,0x03,0x0C,0x05,0x0E,0x02,0x09},//A

{0x0F,0x03,0x0E,0x0B,0x06,0x08,0x04,0x01,0x02,0x0C,0x0D,0x0A,0x05,0x00,0x09,0x07},//B

{0x08,0x0C,0x07,0x0E,0x0A,0x01,0x06,0x02,0x0F,0x04,0x0D,0x03,0x00,0x0B,0x05,0x09},//C

{0x02,0x0E,0x0A,0x07,0x09,0x06,0x04,0x08,0x00,0x01,0x05,0x0C,0x0D,0x0B,0x0F,0x03},//D

{0x0E,0x00,0x0B,0x05,0x0C,0x0D,0x0F,0x06,0x04,0x08,0x01,0x0A,0x09,0x07,0x03,0x02},//E

{0x01,0x0F,0x03,0x07,0x06,0x00,0x0D,0x0C,0x02,0x09,0x0E,0x05,0x08,0x0A,0x0B,0x04},//F

};

Select the key by accumulating and taking the lower four bits by Serial Number

For example, SN is  "0123456789123"

The key is Key[keytype][16].

S um = 0x30+0x31+0x32+0x33+0x34+0x35+0x36+0x37+0x38+0x39+0x31+0x32+0x33

Sum = 0x02A3

keytype= Sa & 0x000F                 keytype= 0x0003

Key[keytype][16] = Key[3][16] =

{0x01,0x06,0x0F,0x09,0x0A,0x02,0x04,0x0D,0x07,0x03,0x08,0x00,0x05,0x0B,0x0C,0x0E},//3

/*Encryption*/

/**********************************************************************************

*Parameter *tabIn requires the first address of the encrypted data (plaintext).

*Parameter *tabOut stores the encrypted data address (ciphertext).

*The parameter len requires the length of the encrypted data

*Parameter type key selection

**********************************************************************************/

void encrypt(unsigned char *tabIn,unsigned char *tabOut,unsigned char len,unsigned char type)

{

unsigned char i=0;

unsigned char codeL,codeH;

for(i=0;i<len;i++)

{

codeL = tabIn[i]&0x0F;

codeH = (tabIn[i]&0xF0)>>4;

tabOut[i] = 0;

tabOut[i] |= key[type][codeL];

tabOut[i] |= (key[type][codeH]<<4);

}

}

/*decrypt*/

/**********************************************************************************

*Parameter *tabInThe  first address (ciphertext) of the data to be decrypted

*Parameter *tabOut stores the decrypted data address (plaintext).

*The parameter len requires decrypted data length

*Parameter type key selection

**********************************************************************************/

void decrypt(unsigned char *tabIn,unsigned char *tabOut,unsigned char len,unsigned char type)

{

unsigned char i=0,j=0;

unsigned char codeL,codeH;

for(i=0;i<len;i++)

{

codeL = tabIn[i]&0x0F;

codeH = (tabIn[i]&0xF0)>>4;

tabOut[i] = 0;

for(j=0;j<16;j++)

{

if(codeL==key[type][j])

{

tabOut[i] |= j;

}

if(codeH==key[type][j])

{

tabOut[i] |= (j<<4);

}

}

}

}

Example:

Send mobile phone to read skateboard information instruction code:

Before encryption: F0 B0  05 C4 30

Encrypted with the key Key[3][16].

After encryption: E1 01 12 5A 91

Description of the encryption bytes

- Device Information's Serial Number is not encrypted

- When a VCU is upgraded, the packet number of the upgraded data is not encrypted

- When the VCU is upgraded, the number of bytes sent or replied to is less than 3 bytes, and it is not encrypted

In addition to the above points, all other data needs to be encrypted.